UAV-SEAD Dataset Investigation & Processing Guide

Chapter 1 – Introduction

1.1 What is UAV-SEAD?

The UAV-SEAD (UAV Sensor Error and Anomaly Detection) dataset is a large-scale UAV telemetry dataset developed for research in fault detection, anomaly detection, health monitoring, and autonomous flight analytics. The dataset contains telemetry collected from drones operating on the PX4 flight controller, one of the most widely used open-source autopilot systems for unmanned aerial vehicles (UAVs).

Unlike traditional datasets that contain only images or numerical tables, UAV-SEAD stores complete flight telemetry generated during real UAV missions. Every flight log records information from multiple onboard sensors such as GPS, IMU, battery, barometer, optical flow sensor, estimator, actuator outputs, and several other flight controller modules.

Each flight is stored as a ULog (.ulg) file, which acts as the flight recorder of the UAV. These logs preserve the complete state of the aircraft throughout the mission, from take-off to landing.

1.2 Why was UAV-SEAD created?

Modern UAVs depend on many sensors working together to estimate their position, orientation, velocity, battery health, and environmental conditions. A failure in any one of these sensors can lead to incorrect navigation decisions and may ultimately result in mission failure.

The UAV-SEAD dataset was created to provide researchers with a standardized collection of UAV flight logs that can be used to develop and evaluate intelligent algorithms for:

- Fault detection

- Sensor anomaly detection

- Predictive maintenance

- Flight monitoring

- Autonomous health assessment

- AI-assisted decision support

The dataset allows researchers to study how different sensors behave during normal and abnormal flight conditions and provides annotated flight logs for evaluating anomaly detection techniques.

1.3 Why did we choose UAV-SEAD?

Our project aims to develop an AI-assisted telemetry analytics and mission risk assessment system for tactical multirotor ISR UAVs.

To achieve this objective, we required a dataset that satisfies the following requirements:

- Large number of real UAV flights

- Raw telemetry instead of only processed statistics

- Multiple onboard sensors

- Publicly available for academic research

- Suitable for machine learning

- Compatible with PX4 autopilot

- Includes anomalous flight data

The UAV-SEAD dataset satisfies all these requirements and therefore serves as the primary dataset for our project.

Another major advantage is that the dataset stores complete telemetry rather than pre-engineered features. This allows us to design our own preprocessing pipeline, feature engineering techniques, and machine learning workflow instead of relying on predefined data representations.

1.4 Dataset Specifications

The UAV-SEAD dataset has the following characteristics.

| Property | Description |
| --- | --- |
| Dataset Name | UAV-SEAD |
| Approximate Size | ~30 GB |
| Flight Controller | PX4 Autopilot |
| Raw Flight Logs | 3196 |
| Annotated Flight Logs | 1396 |
| Total Flight Time | More than 52 hours |
| Data Format | ULog (.ulg) |
| Number of Telemetry Topics | Up to 84 (varies between flights) |
| Main Purpose | UAV anomaly detection and telemetry analytics |

Although the dataset contains up to 84 telemetry topics, not every flight records all topics. During our initial investigation, we successfully loaded one flight log that contained 30 telemetry topics, demonstrating that the available telemetry depends on the sensors enabled and the logging configuration of that specific flight.

1.5 How UAV-SEAD fits into our project

The UAV-SEAD dataset forms the foundation of our entire project.

Rather than training a machine learning model directly on raw flight logs, the dataset will pass through a complete preprocessing pipeline before becoming suitable for machine learning.

The overall workflow of our project is shown below.

UAV-SEAD Dataset

│

▼

Raw ULog Flight Logs

│

▼

Telemetry Extraction

│

▼

Data Cleaning & Preprocessing

│

▼

Common Telemetry Schema

│

▼

Feature Engineering

│

▼

Machine Learning Model

│

▼

Mission Risk Assessment

│

▼

Ground Control Dashboard

The UAV-SEAD dataset therefore acts as the source of raw telemetry from which all subsequent stages of the project are developed. Every machine learning feature, anomaly prediction, and mission risk assessment will ultimately originate from these raw flight logs.

Chapter 2 – Overall Dataset Structure

After downloading the dataset from Hugging Face, we observed the following top-level directory structure:

Dataset/

│

├── ulg_files/

├── mapping.json

├── README.md

└── .cache/

Each component has a specific purpose.

2.1 ulg_files/

This is the most important directory in the dataset.

It contains all raw UAV flight logs recorded using the PX4 flight controller. Every .ulg file inside this directory represents one complete UAV flight.

All telemetry used in our project—including GPS, IMU, battery, actuator outputs, EKF data, and vehicle status—is extracted from these flight logs.

This folder serves as the primary data source for our machine learning pipeline.

2.2 mapping.json

The mapping.json file acts as an index for the dataset.

Instead of storing telemetry, it contains information that links flight logs with their corresponding annotations and metadata. During later stages of the project, this file will be used to identify which flights contain anomalies and how they are labelled.

The raw .ulg files themselves remain unchanged; mapping.json provides the additional information needed to interpret them.

2.3 README.md

The README file contains the official documentation for the dataset.

It describes:

- Dataset overview

- Dataset statistics

- Folder organization

- Number of flight logs

- Number of annotated flights

- Telemetry topics

- Citation information

Before beginning implementation, this document served as our primary reference for understanding the dataset.

2.4 .cache/

The .cache directory is automatically created by Hugging Face during the download process.

It stores temporary download information and cached files.

This directory is not used during preprocessing, feature engineering, or machine learning, and can be ignored for the purposes of this project.

Key Observation

From our investigation, we concluded that only the ulg_files directory contains the raw telemetry required for machine learning. The remaining files either provide documentation (README.md), annotations (mapping.json), or download management (.cache).

Chapter 3 – Understanding ulg_files

The ulg_files directory is the most important component of the UAV-SEAD dataset. It contains all the raw flight logs that were recorded by UAVs during their missions using the PX4 autopilot. Every machine learning feature, anomaly label, and mission risk assessment in our project will ultimately originate from the data stored in this directory.

Unlike a traditional tabular dataset, the telemetry is not stored in CSV or Excel files. Instead, it is organized into a hierarchical folder structure where each .ulg file represents one complete UAV flight.

3.1 Directory Organization

During our investigation, we discovered that the dataset follows two different directory organizations.

Structure 1 – Date → Flight Logs

The first and simpler structure is organized directly by the flight date.

ulg_files/

│

├── 2018-07-09/

│      ├──18_55_04.ulg

│      ├──18_56_32.ulg

│      ├──19_03_09.ulg

│      ├──19_12_45.ulg

│      └──...

│

├──2018-07-10/

│      ├──...

│

└──...

In this structure:

- The folder name represents the date on which the flights were conducted.

- Every .ulg file inside the folder represents one complete flight.

- The filename is based on the time at which the log was created.

For example,

2018-07-09/

│

└──18_55_04.ulg

can be interpreted as

Flight recorded on 9 July 2018 at approximately 18:55:04.

Each file contains the complete telemetry from take-off to landing.

Structure 2 – Date → Session → Flight Logs

While exploring the dataset further, we discovered another organization.

ulg_files/

│

└──2018-08-19/

│

├──sess080/

│      ├──log001.ulg

│      ├──log002.ulg

│      └──...

│

├──sess081/

│      ├──log001.ulg

│      └──...

Here an additional session folder appears.

The hierarchy becomes

Date

↓

Session

↓

Flight Log

↓

ULog

3.2 Why Do Both Structures Exist?

During our investigation, we initially thought the dataset followed only the first structure. However, after exploring additional folders, we discovered that some dates contain session directories while others do not.

Based on our observations, the dataset appears to have been collected under different experimental conditions.

For ordinary flight recordings, the hierarchy is simply:

Date

↓

Flight Logs

For organized experimental campaigns, multiple flights appear to have been grouped into sessions.

A session most likely represents a collection of flights performed during the same experiment, test sequence, or data collection activity.

For example,

2018-08-19

↓

sess080

↓

log001.ulg

log002.ulg

log003.ulg

may represent several flights that belong to the same testing session.

Important Note:
The dataset documentation available to us does not explicitly describe the purpose of these session folders. Therefore, the interpretation that a session groups related flights is based on the observed directory structure rather than an explicit statement from the dataset authors.

3.3 What Does This Mean for Our Project?

The existence of two folder structures has an important implication for preprocessing.

Instead of assuming that all flight logs exist directly inside a date folder, our preprocessing pipeline must recursively search for every .ulg file.

Our extraction algorithm should therefore follow this logic:

Open ulg_files/

↓

Visit every folder

↓

If another folder exists

↓

Enter it

↓

Repeat

↓

If .ulg file found

↓

Process Flight

This approach ensures that every flight log is discovered regardless of whether it is stored directly inside a date folder or inside a session folder.

3.4 One Flight = One ULog

The most important conclusion from this investigation is:

Every .ulg file represents one complete UAV flight.

Regardless of where it is stored,

18_55_04.ulg

and

sess080/log001.ulg

are both complete flight recordings.

Each contains telemetry beginning at flight initialization and ending when logging stops after landing.

Therefore, throughout this project, the flight—not the folder—is the fundamental unit of data.

Key Takeaways

- ulg_files is the primary source of raw telemetry.

- The dataset contains two folder organizations:

  - Date → Flight

  - Date → Session → Flight

- Session folders appear to group related flight logs.

- Every .ulg file corresponds to one complete UAV mission.

- Our preprocessing pipeline must recursively search for all .ulg files instead of assuming a fixed folder structure.

Chapter 4 – What is a ULog?

Understanding the ULog format is essential because it is the starting point of our entire machine learning pipeline.

Unlike datasets stored as CSV files, SQL databases, or spreadsheets, UAV-SEAD stores its information in the ULog format developed for the PX4 autopilot.

4.1 What is PX4 Logging?

PX4 is an open-source autopilot system used in drones and autonomous vehicles.

During every flight, the flight controller continuously records the state of the UAV by collecting data from numerous onboard sensors and internal software modules.

This continuous recording process is called logging.

Logging begins shortly before take-off and continues until the flight ends.

Throughout the flight, the autopilot records information such as:

- GPS position

- Battery voltage

- IMU measurements

- Vehicle attitude

- Motor outputs

- Sensor health

- Estimator information

- Flight mode

- RC commands

- CPU usage

- System status

These measurements are stored together in a single log file.

4.2 ULog as the UAV's Black Box

An aircraft uses a Flight Data Recorder (Black Box) to store everything that happens during a flight.

Similarly, a PX4-based drone stores its operational data inside a ULog file.

For this reason, a ULog can be considered the black box recorder of a UAV.

Just as investigators analyze an aircraft's black box after an incident, researchers analyze ULog files to understand UAV behavior, detect anomalies, and evaluate flight performance.

4.3 One Flight = One ULog

One of the most important concepts established during our investigation is:

One ULog file corresponds to one complete UAV flight.

For example,

18_55_04.ulg

contains:

- Vehicle initialization

- Sensor startup

- Take-off

- Entire flight mission

- Landing

- Shutdown

Everything that occurred during that mission is stored inside a single file.

4.4 Binary File Format

A ULog file is not a human-readable text file.

It is stored as a binary file.

Unlike a CSV file,

Timestamp,Voltage

0,25.1

1,24.9

a ULog file contains encoded binary data that cannot be interpreted directly.

Therefore, specialized software such as PyULog is required to decode the binary data into readable telemetry.

This is why we first installed the pyulog library before beginning our investigation.

4.5 Topics

Instead of storing one giant table, a ULog organizes information into topics.

A topic represents one category of telemetry.

Examples include:

- battery_status

- vehicle_gps_position

- sensor_combined

- vehicle_attitude

- estimator_status

- actuator_outputs

Each topic records only information relevant to that subsystem.

4.6 Telemetry Streams

Each topic continuously records measurements throughout the flight.

Conceptually, the ULog looks like this:

GPS

│

IMU

│

Battery

│

Barometer

│

Vehicle Attitude

│

EKF

│

Actuator Outputs

│

Vehicle Status

│

CPU Load

│

Telemetry Status

│

▼

ULog

Each sensor generates its own stream of data, and the ULog stores all of these streams together in a synchronized flight log.

4.7 Why Is This Important?

Our machine learning model will never read the binary ULog directly.

Instead, our processing pipeline will follow these steps:

ULog

↓

Decode using PyULog

↓

Extract Topics

↓

Convert Topics into DataFrames

↓

Preprocess

↓

Feature Engineering

↓

Machine Learning Dataset

Thus, the ULog serves as the raw source from which all future processing begins.

Key Takeaways

- ULog is the native logging format of PX4.

- It functions as the UAV's black box recorder.

- One ULog stores one complete flight.

- ULog files are binary and require decoding.

- Telemetry is organized into independent topics.

- These topics form the input to our preprocessing pipeline.

Chapter 5 – Flight Investigation

After understanding the structure of the dataset and the ULog format, our next objective was to inspect the contents of an individual flight log.

Rather than processing the entire 30 GB dataset immediately, we decided to begin with a single flight. This allowed us to understand the structure of the telemetry before designing a preprocessing pipeline for all flight logs.

5.1 Loading the First Flight

Using the PyULog library, we successfully loaded one ULog file.

The first observation was that the flight contained:

30 telemetry topics

This confirmed that a single ULog file stores multiple independent streams of telemetry collected throughout the flight.

5.2 Initial Topic Investigation

The first flight contained topics such as:

- battery_status

- vehicle_gps_position

- vehicle_local_position

- vehicle_attitude

- actuator_outputs

- sensor_combined

- estimator_status

- ekf2_innovations

- optical_flow

- telemetry_status

- system_power

These topics represent different subsystems of the UAV, including power management, navigation, inertial sensing, estimation, communication, and system health.

To simplify our investigation, we grouped them into categories:

| Category | Example Topics |
| --- | --- |
| Power | battery_status, system_power |
| Navigation | vehicle_gps_position, vehicle_local_position |
| Attitude | vehicle_attitude, vehicle_attitude_setpoint |
| Sensors | sensor_combined, sensor_baro, optical_flow |
| Estimation | estimator_status, ekf2_innovations |
| Flight Control | actuator_controls_0, actuator_outputs |
| Communication | telemetry_status, input_rc |
| System Health | cpuload, vehicle_status_flags |

This categorisation provides a clearer understanding of how different components of the UAV contribute telemetry during flight.

5.3 Why Does the Number of Topics Differ Between Flights?

During our investigation, we observed that one flight contained 30 topics, whereas the dataset documentation indicates that the complete UAV-SEAD dataset can contain up to 84 topics.

This difference is expected and can occur for several reasons:

- Some sensors may not be installed on every UAV.

- Certain sensors may be disabled for specific flights.

- The PX4 logging configuration may differ between missions.

- Different flight modes or experiments may enable different telemetry streams.

- Firmware versions may introduce or remove specific topics.

Therefore, not every ULog file contains the same set of topics. Our preprocessing pipeline must be designed to handle this variability instead of assuming that every flight has identical telemetry.

Key Takeaways

- We began our investigation with a single flight rather than the entire dataset.

- The first flight contained 30 telemetry topics.

- Topics represent different UAV subsystems.

- The number of available topics varies between flights due to differences in hardware, configuration, and mission setup.

- A flexible preprocessing pipeline is essential to accommodate these variations.

Chapter 6 – Understanding PX4 Topics

A ULog file is not stored as a single spreadsheet. Instead, it is composed of multiple topics, each representing a specific source of telemetry generated during flight.

Understanding the relationship between topics, messages, and fields is essential before performing any preprocessing or feature engineering.

6.1 What is a Topic?

A topic is a named stream of telemetry that represents information from a particular sensor or subsystem of the UAV.

Each topic records measurements related to one specific function.

Examples include:

- battery_status

- vehicle_gps_position

- sensor_combined

- vehicle_attitude

- estimator_status

Each topic is independent and maintains its own sequence of measurements over time.

6.2 What is a Message?

A message is a single observation recorded within a topic at a particular timestamp.

For example, the battery_status topic may record one message every few hundred milliseconds.

A single message could look conceptually like this:

| Field | Value |
| --- | --- |
| timestamp | 10,500,000 |
| voltage_v | 24.8 |
| current_a | 12.6 |
| remaining | 0.82 |
| warning | 0 |

As the flight progresses, many such messages are recorded, forming the complete telemetry stream for that topic.

6.3 What is a Field?

A field is an individual variable within a message.

Each field describes one measurable property.

For the battery_status topic, common fields include:

battery_status

│

├── timestamp

├── voltage_v

├── current_a

├── remaining

└── warning

Here:

- timestamp indicates when the measurement was recorded.

- voltage_v stores the battery voltage in volts.

- current_a stores the battery current in amperes.

- remaining represents the estimated battery charge (0 to 1).

- warning indicates the battery warning level reported by PX4.

6.4 Relationship Between Topics, Messages, and Fields

The structure can be visualized as follows:

ULog

│

├── Topic (battery_status)

│      ├── Message 1

│      ├── Message 2

│      ├── Message 3

│      └── ...

│

├── Topic (vehicle_gps_position)

│      ├── Message 1

│      ├── Message 2

│      └── ...

│

└── Topic (sensor_combined)

├── Message 1

├── Message 2

└── ...

Each message consists of multiple fields, and together the messages form the complete telemetry history for that topic.

Key Takeaways

- A topic is a stream of related telemetry from a specific subsystem.

- A message is one recorded observation within a topic at a particular time.

- A field is an individual variable contained within a message.

- A ULog file is composed of many topics, each with its own sequence of messages and fields.

- This hierarchical structure forms the foundation for all subsequent preprocessing, feature engineering, and machine learning tasks in our project.

Chapter 7 – Important Telemetry Subsystems

This chapter introduces the major telemetry subsystems used by a multirotor UAV and explains their significance in flight monitoring and mission risk assessment.

7.1 Battery System

Purpose

The Battery System monitors the electrical health of the UAV and supplies power to all onboard components. It is essential for determining flight endurance and preventing power-related failures.

Important Topics

- battery_status

- system_power

Why It Is Important

- Flight duration

- Remaining energy

- Safe landing decisions

- Battery degradation monitoring

Possible Anomalies

- Rapid voltage drop

- Excessive current draw

- Sudden battery failure

- Low battery warning

Importance in Mission Risk Assessment

Battery failure can immediately threaten mission completion and aircraft safety.

7.2 Navigation System

Purpose

The Navigation subsystem determines the UAV's position, altitude, and velocity throughout the mission.

Important Topics

- vehicle_gps_position

- vehicle_local_position

- vehicle_global_position

- home_position

- vehicle_land_detected

Why It Is Important

- Path following

- Return-to-home

- Mission execution

- Position estimation

Possible Anomalies

- GPS loss

- GPS spoofing

- Position jumps

- Large position drift

- Altitude deviation

7.3 IMU & Sensors

Purpose

The IMU continuously measures acceleration and angular velocity required for stable flight.

Important Topics

- sensor_combined

- sensor_accel

- sensor_gyro

- sensor_mag

- sensor_baro

Why It Is Important

- Aircraft stabilization

- Motion estimation

- Orientation estimation

Possible Anomalies

- Sensor bias

- High vibration

- IMU failure

- Magnetometer interference

- Barometer drift

7.4 Flight Dynamics

Purpose

Represents how the aircraft moves and how the controller attempts to stabilize it.

Important Topics

- vehicle_attitude

- vehicle_attitude_setpoint

- vehicle_rates_setpoint

- actuator_controls_0

- actuator_outputs

Why It Is Important

- Aircraft stability

- Control accuracy

- Motor response

Possible Anomalies

- Oscillation

- Motor saturation

- Excessive roll/pitch

- Control instability

7.5 Communication

Purpose

Monitors communication between the UAV and Ground Control Station.

Important Topics

- telemetry_status

Possible Anomalies

- Packet loss

- Signal degradation

- Communication timeout

7.6 Estimator

Purpose

The estimator combines sensor information to estimate the UAV's true state.

Important Topics

- estimator_status

- ekf2_innovations

Possible Anomalies

- EKF divergence

- Innovation spikes

- Sensor inconsistency

- Estimator reset

7.7 Environment

Purpose

Represents external environmental conditions affecting flight.

Important Topics

- wind_estimate

- distance_sensor

- optical_flow

Possible Anomalies

- Strong crosswind

- Unexpected obstacles

- Poor optical flow quality

7.8 Mission

Purpose

Stores mission execution and flight mode information.

Important Topics

- vehicle_command

- vehicle_control_mode

- input_rc

Possible Anomalies

- Unexpected mode switching

- Incorrect pilot commands

- Mission interruption

7.9 System Health

Purpose

Monitors the internal operational condition of the flight controller.

Important Topics

- cpuload

- vehicle_status_flags

- actuator_armed

Possible Anomalies

- CPU overload

- System fault

- Arming failure