(Note: This is an old methodology, proposed before exploring the dataset; it is just to give an overview)

Stage 1 – Telemetry Data Acquisition

The first stage of the proposed system involves collecting telemetry data generated by the UAV during flight. Every modern UAV continuously produces sensor information that describes its position, movement, health, communication status, and overall operating condition.

For this project, telemetry data will be obtained from the PX4 flight controller ecosystem. Historical flight logs from the UAV-SEAD dataset will be used for training and testing machine learning models, while PX4 Software-in-the-Loop (SITL) simulation will be used to generate live telemetry for real-time system demonstration.

The collected telemetry includes information such as GPS coordinates, altitude, speed, roll, pitch, yaw, battery voltage, battery current, remaining battery percentage, IMU measurements, motor RPM, ESC status, communication signal strength, flight mode, vibration levels, and various system status parameters.

Since the project focuses on Tactical Multirotor ISR UAVs, only telemetry relevant to multirotor platforms will be considered.

Stage 2 – Telemetry Pre-processing

Raw telemetry data cannot be directly used for analysis because different sensors produce values in different formats, units, and sampling rates. Therefore, the collected telemetry is first cleaned and pre-processed.

During this stage, missing values are handled, duplicate records are removed, incorrect sensor readings are filtered, and timestamps are synchronized so that all telemetry parameters correspond to the same point in time.

Sensor values are also normalized to ensure that parameters with different numerical ranges can be analysed together without introducing bias into the machine learning model.

The cleaned telemetry is then organised into a structured format suitable for further analysis.

Stage 3 – Common Telemetry Schema

Different UAV platforms often generate telemetry using different formats and parameter names. To make the system independent of a particular hardware platform, all processed telemetry is converted into a Common Telemetry Schema.

The Common Telemetry Schema groups telemetry into logical subsystems, including:

- Mission Information

- Navigation

- Flight Dynamics

- IMU

- Power System

- Propulsion System

- Communication System

- System Health

This unified structure allows telemetry from different sources to be analysed using the same processing pipeline and also simplifies future expansion to additional UAV platforms.

Stage 4 – Feature Engineering

After the telemetry has been converted into a common format, meaningful features are extracted for machine learning analysis.

Instead of analysing only individual sensor values, the system derives additional information that better represents the condition of the UAV. Examples include battery discharge rate, communication stability, vibration trends, altitude variation, velocity changes, motor performance consistency, and IMU stability.

These engineered features provide a more informative representation of UAV behaviour and improve the performance of anomaly detection algorithms.

4.6 Stage 5 – Telemetry Analytics

The extracted features are then analysed to understand the current operating condition of the UAV.

Telemetry analytics involves continuously monitoring relationships among multiple onboard systems rather than observing individual sensors separately. For example, a sudden drop in battery voltage together with increased motor current and excessive vibration may indicate a developing hardware problem that cannot be identified by analysing a single sensor alone.

This stage provides a comprehensive understanding of the UAV's operational health before machine learning is applied.

4.7 Stage 6 – Machine Learning-Based Anomaly Detection

Once telemetry analytics has been completed, the processed data is passed to the anomaly detection module.

The machine learning model is trained using historical flight logs obtained from the UAV-SEAD dataset. During training, the model learns the characteristics of normal flight behaviour and identifies patterns associated with abnormal operating conditions.

During live operation, incoming telemetry is continuously analysed by the trained model. If the observed telemetry deviates significantly from expected behaviour, the system classifies the event as a potential anomaly.

Examples of detectable anomalies include abnormal battery behaviour, communication degradation, excessive vibration, sensor inconsistencies, navigation errors, abnormal flight dynamics, or unusual motor performance.

The anomaly detection model forms the intelligence layer of the proposed system and enables automatic identification of problems without requiring constant human monitoring.

4.8 Stage 7 – Mission Risk Assessment

Detecting an anomaly alone is often insufficient because not every anomaly represents the same level of operational risk.

Therefore, the detected anomalies are further evaluated by the Mission Risk Assessment Engine.

The risk engine considers several factors, including:

- Number of detected anomalies

- Severity of anomalies

- Mission phase

- Battery condition

- Communication quality

- Navigation reliability

- Overall system health

Based on these factors, the system calculates an overall mission risk score and classifies the current mission into one of four categories:

- Low Risk

- Medium Risk

- High Risk

- Critical Risk

This provides operators with an immediate understanding of the seriousness of the situation instead of requiring them to interpret numerous telemetry values manually.

4.9 Stage 8 – Explainable Decision Support

To improve transparency, the system includes an Explainable Decision Support module.

Instead of displaying only the calculated risk level, the system explains why a particular warning has been generated.

For example, the system may indicate that the mission risk is high because battery voltage is decreasing rapidly, communication signal strength has weakened, and abnormal vibration has been detected simultaneously.

Based on the identified conditions, the system also recommends suitable actions such as:

- Continue Mission

- Reduce Flight Speed

- Return to Base (RTB)

- Perform Emergency Landing

- Inspect Hardware after Landing

These recommendations assist mission operators in making faster and more informed decisions during flight.

4.10 Stage 9 – Ground Control Dashboard

The processed information is presented to the operator through an interactive Ground Control Dashboard.

The dashboard displays live telemetry, flight location, mission route, battery status, communication quality, anomaly alerts, mission risk level, and system health using an intuitive graphical interface.

Instead of overwhelming the operator with raw sensor values, the dashboard presents summarized operational information that is easier to understand and interpret during real-time missions.

The dashboard also includes interactive charts, maps, historical trends, and visual alerts to improve situational awareness.

4.11 Stage 10 – Mission History and Report Generation

After completion of the mission, all telemetry data, detected anomalies, mission risk levels, operator decisions, and flight summaries are stored in a mission history database.

These records support post-flight analysis, comparison between different missions, identification of recurring faults, and long-term performance evaluation.

The system also generates downloadable mission reports in PDF or CSV format containing telemetry summaries, anomaly timelines, mission statistics, and operational recommendations.

These reports can assist operators, maintenance teams, and defence analysts in evaluating overall mission performance.

4.12 Future Expansion

Although the current implementation focuses on Tactical Multirotor ISR UAV operations, the proposed architecture has been designed to support future extensions.

The modular design allows integration with additional UAV platforms, multiple drone fleets, predictive maintenance systems, Edge AI devices such as NVIDIA Jetson, and autonomous mission management frameworks.

As future datasets become available, the machine learning model can also be retrained to improve detection accuracy and support new operational scenarios.

Overall Methodology Flow

Historical Flight Logs (UAV-SEAD)

│

▼

Live Telemetry (PX4 SITL)

│

▼

Telemetry Data Acquisition

│

▼

Telemetry Pre-processing

│

▼

Common Telemetry Schema

│

▼

Feature Engineering

│

▼

Telemetry Analytics

│

▼

Machine Learning Anomaly Detection

│

▼

Mission Risk Assessment

│

▼

Explainable Decision Support

│

▼

Ground Control Dashboard

│

▼

Mission History & Report Generation

│

▼

Future Learning & System Improvement

Why do we need a Common Telemetry Schema?

Imagine we directly use the datasets.

PX4

vehicle_gps_position.lat

ArduPilot

GPS.Lat

UAV-SEAD

latitude

SITL

GLOBAL_POSITION_INT.lat

All four represent the same thing.

But if our AI sees four different names,

it thinks they are four different variables.

Instead we convert everything into

Latitude

Now every source becomes compatible.

This is why companies like Boeing, Airbus, and DRDO standardize interfaces before developing analytics.

Our System Architecture

PX4

UAV-SEAD

PX4 SITL

│

▼

Telemetry Mapper

│

▼

COMMON TELEMETRY SCHEMA

│

▼

Feature Engineering

│

▼

AI Models

│

▼

Mission Risk Engine

│

▼

Dashboard

Notice something.

Our AI will never directly read PX4 logs.

It will only read our schema.

Designing the Schema

I suggest dividing telemetry into seven engineering subsystems.

1. Mission Information

2. Navigation

3. Flight Dynamics

4. IMU

5. Power System

6. Propulsion

7. Communication

8. System Health

This is how aerospace engineers usually organize avionics.

Section 1 — Mission Information

These fields identify the mission.

| Field | Unit | Mandatory | Purpose |
| --- | --- | --- | --- |
| Mission ID | String | ✅ | Unique mission identifier |
| Drone ID | String | ✅ | UAV identification |
| Timestamp | ms | ✅ | Synchronize telemetry |
| Flight Mode | Enum | ✅ | Auto, RTL, Manual, Loiter |
| Mission Phase | Enum | Optional | Takeoff, Patrol, Return, Landing |

These fields are useful for:

- Mission replay

- Timeline analysis

- Report generation

Section 2 — Navigation

This answers:

Where is the drone?

| Field | Unit | Mandatory | Why? |
| --- | --- | --- | --- |
| Latitude | Degrees | ✅ | Live position |
| Longitude | Degrees | ✅ | Live position |
| Relative Altitude | m | ✅ | Flight monitoring |
| Absolute Altitude | m | Optional | Terrain comparison |
| Ground Speed | m/s | ✅ | Mission progress |
| Vertical Speed | m/s | Optional | Climb rate |
| Heading | Degrees | ✅ | Flight direction |
| Satellite Count | Integer | ✅ | GPS reliability |
| HDOP | Float | Optional | GPS accuracy |

Section 3 — Flight Dynamics

How is the UAV moving?

| Field | Unit |
| --- | --- |
| Roll | Degrees |
| Pitch | Degrees |
| Yaw | Degrees |
| Roll Rate | °/s |
| Pitch Rate | °/s |
| Yaw Rate | °/s |

These detect

- unstable flight

- aggressive maneuvers

- oscillations

Section 4 — IMU

This is one of the most important sections.

| Field | Unit |
| --- | --- |
| Acc X | m/s² |
| Acc Y | m/s² |
| Acc Z | m/s² |
| Gyro X | rad/s |
| Gyro Y | rad/s |
| Gyro Z | rad/s |
| Mag X | Gauss |
| Mag Y | Gauss |
| Mag Z | Gauss |

Why?

Motor failures,

wind,

propeller damage,

all affect the IMU.

Section 5 — Power System

Probably the most important subsystem for mission risk.

| Field | Unit |
| --- | --- |
| Battery Voltage | Volts |
| Battery Current | Amps |
| Battery Remaining | % |
| Battery Temperature | °C |
| Estimated Remaining Time | Minutes |

Useful for

Return-to-Base calculations.

Section 6 — Propulsion

Very important.

| Field | Unit |
| --- | --- |
| Motor 1 RPM | RPM |
| Motor 2 RPM | RPM |
| Motor 3 RPM | RPM |
| Motor 4 RPM | RPM |
| ESC Temperature | °C |
| ESC Current | Amps |
| Vibration Level | m/s² or RMS |

This section will drive

Predictive Maintenance.

Section 7 — Communication

Can the operator still communicate?

| Field | Unit |
| --- | --- |
| RSSI | % or dBm |
| Link Quality | % |
| Packet Loss | % |
| Latency | ms |

Communication failure is often a precursor to mission failure.

Section 8 — System Health

High-level system status.

| Field | Type |
| --- | --- |
| EKF Status | Boolean / Enum |
| GPS Status | Enum |
| IMU Status | Enum |
| Battery Status | Enum |
| Motor Status | Enum |
| Failsafe Active | Boolean |
| Error Code | Integer |

Instead of looking at 100 sensor values,

operators can immediately see subsystem health.

Phase 1 — Telemetry Acquisition

Objective

Collect telemetry from one or more UAV data sources.

Possible sources

- PX4 Flight Logs

- ArduPilot Logs

- UAV-SEAD Dataset

- PX4 SITL

- Custom Simulation

Output

Raw Telemetry

Think of this as collecting raw sensor readings.

Phase 2 — Data Preprocessing

Raw telemetry is messy.

Different datasets

Different formats.

Different units.

Different sampling rates.

Example

PX4

Battery Remaining

0.82

Another dataset

Battery

82%

Both represent

82%

So preprocessing will

- Clean

- Normalize

- Synchronize timestamps

- Handle missing values

- Remove duplicate records

Output

Clean Telemetry

Phase 3 — Common Telemetry Schema

This is the heart of the project.

Every source

PX4

ArduPilot

UAV-SEAD

↓

becomes

Latitude

Longitude

Roll

Pitch

Yaw

Battery Voltage

Battery Current

RSSI

Motor RPM

...

One common format.

This makes

our AI

independent of the source.

Phase 4 — Feature Engineering

This is where

raw telemetry

becomes

meaningful information.

Example

Raw

Voltage

15.8V

Feature

Voltage Drop Rate

Raw

Motor RPM

Feature

RPM Imbalance

Raw

GPS

Feature

GPS Drift

Raw

Battery %

Feature

Estimated Remaining Flight Time

This step creates intelligence from raw numbers.

Phase 5 — Telemetry Analytics

Now

instead of

looking at one sensor,

the system looks at

everything together.

Example

Battery

GPS

Motor

IMU

↓

Mission Health

This is the core analytics engine.

Phase 6 — Anomaly Detection

Now the system asks

Is anything unusual?

Examples

Battery discharge

too fast.

Motor RPM

too low.

GPS suddenly jumps.

RSSI drops.

Vibration increases.

Flight mode changes unexpectedly.

Output

Detected Anomalies

Phase 7 — Mission Risk Assessment

This is where

our project becomes unique.

Instead of

only saying

Motor Fault

the system asks

Can the mission still continue safely?

Example

Battery

↓

Motor

↓

RSSI

↓

GPS

↓

Overall Risk

↓

HIGH

This is

Mission Risk Assessment.

Phase 8 — Explainable Decision Support

The system should never say

Risk = High

without explanation.

Instead

Mission Risk

HIGH

Reason

Battery dropping rapidly

Motor 2 RPM unstable

RSSI below threshold

Recommendation

Return-to-Base

This is exactly what operators need.

Phase 9 — Dashboard & Visualization

Now everything becomes visual.

Pages

Overview

↓

Live Monitor

↓

Map

↓

Mission History

↓

Reports

↓

Analytics

Phase 10 — Mission Database

Every flight

gets stored.

Later

we can compare

Mission A

vs

Mission B

Find trends

Generate reports

Improve AI